Premises Dept , 2nd     Floor , New Building

Indian bank

Hazratganj, Lucknow

BID DOCUMENT -

CIVIL, INTERIOR & ELECTRICAL WORK (INCLUDING DATA CABLING) & TELEPHONE

IN EXISTING PREMISES OF

INDIAN BANK MILKIPUR –LUCKNOW

PART – II FINANCIAL BID

Ref. No. - …………………

Date……………….

Name of the Contractor

…………………………………

………………………………….

………………………………….

| Last date for submission of Bid | 27/02/2026 upto15:00HRS. at Indian Bank, Zonal Office Lucknow, Premises Department, 2ND Floor, New Building, Hazaratganj, Lucknow -226001 |
| --- | --- |
| Date of Opening Tender | 27/02/2026 upto15:30HRS. at Indian Bank, Zonal Office Lucknow, Premises Department, 2ND Floor, New Building, Hazaratganj, Lucknow -226001 (Technical Bid only) |
| Cost of Tender | Free of Cost |

This part of the tender should be submitted in a separate envelope and financial bid shall be quoted only this form and no other form will be accepted.

TOTAL Number of pages 30 Nos

BOQ FOR FURNISHING WORK AT MILKIPUR BRANCH

| S.NO. | DESCRIPTION OF ITEM | UNIT | QUANTITY | RATE (Rs.) | AMOUNT (Rs.) |
| --- | --- | --- | --- | --- | --- |
| 1.0 | SUSPENDED FALSE CEILING |  |  |  |  |
| 1.1 | GYPSUM FALSE CEILING WITH COVE |  |  |  |  |
| 1.0 | Providing, fabricating and erecting in position totally flushed suspended false ceiling with 12.0mm thick Gypsum board, fixed to a metal frame work comprising of ceiling channel of size 80mmx26mmx0.5mm @ every 457cm centre, inter channel 45mmx15mmx0.9mm @ every 1220 cm centres, peri-meter channel 20mmx26 mm all along the wall/partitions, strap hanger suspenders 25mmx0.5mm with nuts and bolts 6.4mmx12.7mm, connecting clips 2.6mm thick, dry walls screws 25m long, soffit cleats 27mmx37mmx25mmx1.6mm thick and dash fastenes / rawl plugs 12.5 mm diax35mm long with 6mm dia bolts etc. complete with recommended fillers, paper tapes and applying two coats of acrylic emulsion paint of approved shade and make over a coat of primer including making necessary cut outs for light fittings/ ACs etc. complete as per manufacture's specifications. (Quantity as per layout plan/site requirement) | Sqft | 400.00 |  |  |
| 1.2 | FALSE CEILING (ARMSTRONG CEILINGS – FINE FISSURED MICROLOOK EDGE WITH SILHOUETTE GRID (BLACK REVEAL)) |  |  |  |  |
|  | Providing and fixing false ceiling system manufactured by M/s. Armstrong World Industries using hot dipped galvanized steel section, rotary stitched main tee of size 15mm x 42 mm web height, having 0.36 mm gauge at every 600 mm centre to centre maximum and rotary stitched cross tee of size 15 mm x 42 mm, having 0.33 mm gauge at every 600 mm. c/c. and wall angle of size 19 x 19 mm., having 0.35 mm gauge fixed to the periphery of the wall. The above grid is suspended at every 600mm c/c. in both directions using 2.0 mm. thick pre-straightened GI wire laying FINE FISSURED MICROLOOK WITH SILHOUETTE GRID(BLACK REVEAL) ceiling tiles manufactured by M/s. Armstrong World Industries, of size 600mmx600mmx15mm having NRC 0.55, Light reflectance of >84% (WT), thermal conductivity k = 0.052-0.057 W/m0K, Humidity Resistance of 99% , having Fire Performance CLASS O / CLASS 1 (BS 476), surface having 3 coats of white paint with Fine Fissured, back of the tile duly sanded and finished with a coat of protective paint. | Sqft | 450.00 |  |  |
| 3.0 | PARTITIONS |  |  |  |  |
| 3.1 | CASH CABIN PARTITIONS : Supplying & Erecting full height solid cash cabin partitions (upto false ceiling as per following specifications : Basic frame work: Framing shall consist of Heavy duty Aluminium Box Sections 50mm x 50mm x 2 mm thick @ 600mm c/c both ways Horizontally and Vertically. The vertical alternate frames to be fixed to the ceiling with necessary fixtures. Vertical members shall be suitably anchored at ceiling level to ensure rigidity. Skin membrane: To be finished with 12 mm thick BWP grade plywood on each face with grooves, as per drawings/ instructions. Finishing: 1.0mm thick laminate of same or combination shades with grooves in both faces as per instructions. Skirting- 100mm high laminate of approved make & shade all as directed. Top Border: 150mm high laminate of approved make & shade all as directed. It shall have decorative aluminum grill (from 1200mm to 2100mm) fixed with teak wood beading (size – 30mm x 20mm, as per profile) with melamine finish. The job shall also include for provision of laying conduits, switch boxes etc. The erection of partition shall include for expansion bolts cleats, clamps bolts, nuts, screws, rivets and other accessories, all complete up to the satisfaction of the Architect. All door opening shall have rebated teak wood framework (size – 75mm x 50mm, as per profile) on vertical (2 Nos.) and horizontal (1 No.) sides, with melamine finish and rebate of 40mm x 12mm, all complete up to the satisfaction of the Architect. Quoted rates shall include all necessary materials, accessories and labour and also to make provisions for electrical / telephone / computer wiring conduits and switch boxes, etc. (Materials for wiring, conduits, switches etc. shall be separately paid for) | Sqft | 70.00 |  |  |
| 3.2 | Supplying and erecting full height partly glazed / unglazed partitions as per following specifications: Basic frame work: Framing shall consist of Heavy duty Aluminum Box Sections 50mm x 50mm x 2mm thick @ 600mm c/c both ways Horizontally and Vertically. The vertical alternate frames to be fixed to the ceiling with necessary fixtures. Vertical members shall be suitably anchored at ceiling level to ensure rigidity. Exposed frame members in the glazed portions shall have wider spacing (approx. 1200 mm) and be finished with 1.00 mm thick laminate of same or combination shades, over 6mm BWP grade ply. Skin membrane: (Opaque portion up to 900mm from floor level and from 2100mm to ceiling, finished with 12 mm thick BWP grade plywood on each face with grooves, as per drawings/ instructions. Finishing: 1.0mm thick laminate of same or combination shades with grooves as per instructions. Glazing: 12mm thick clear float glass fixed with polished teak / steam beech wood glazing beads of approved shape (finished 12 x 25mm). Skirting- 100mm high laminate of approved make & shade all as directed. Top Border: 150mm high laminate of approved make & shade all as directed. Quoted rates shall include all necessary materials, accessories and labour and also to make provisions for electrical / telephone / computer wiring conduits and switch boxes, etc. (Materials for wiring, conduits & switches etc. shall be separately paid for) | Sqft | 150.00 |  |  |
| 3.3 | Supply and erecting low height opaque partitions (up to maximum 1200mm height) conforming to the following specifications : Basic frame work: Framing shall consist of Heavy duty Aluminium Box Sections 50mm x 50mm @ 600mm c/c both ways Horizontally and Vertically. The vertical alternate frames to be fixed to the ceiling with necessary fixtures. Vertical members shall be suitably anchored at ceiling level to ensure rigidity. Skin membrane: 12 mm thick BWP grade ply on each face up to 900mm height. Glazing: 12mm glass in remaining portion, Finishing: 1.0-mm thick laminate of same or combination shades with grooves as per instructions. Edge moulds: All free top/ end surfaces shall have polished teak / steam beech wood moulds of approved shape (from 18 x 50mm sections). Skirting- 100mm high laminate of approved make & shade all as directed. Quoted rates shall include all necessary materials, accessories and labour and also allow provisions for electrical / telephone/computer wiring, conduits and switch boxes, etc. (Materials for wiring, conduits and switch boxes shall be separately paid for.) | Sqft | 45.00 |  |  |
| 3.4 | Supply and Erecting Bison Panel Board Partition for UPS area (UPTO 7' HT). The framework shall consist of Aluminium Box Section as per the specifications mentioned above. The framework shall be than covered with 8mm thick Bison Panel on both the sides and also in the external periphery of the framework. | Sqft | 25 |  |  |
| 4.0 | COUNTER & WORK STATIONS |  |  |  |  |
| 4.1 | Providing and erecting in position Cash Work Station of approx. size 1370x900x750mm (4.5’x3.0’x2.5’), made of 19mm thick block board pasted with 1.00mm laminate inclusive of a 350mm wide customer top at 1155 mm level,. Writing top & customer top should be finished with mirror polished; 18mm thick granite slab (Jet black or shade as directed by Bank's Engineer) with rounded edges and sides with 1.0mm thick laminate of approved shade/make. Both should have sufficient gap in between to accommodate computer monitor up to full depth. The workstations shall be supported from the partition system made of 19mm thick BWP grade block board finished with 1.00 mm thick laminate of approved shade/make. Free edges shall have 18 mm thick, polished white or steam beach wood moldings of half-round profile as per drawings and instructions and 100mm high laminate of approved make & shade all as directed. Each cash workstation shall comprise of the drawer unit, Tea tray at top, Cash drawers with approved telescopic channels for drawer-slides, automatic locks (keys in duplicate), wire managers, space for computer CPU, modesty panel and other necessary hardware and accessories including satin finish S. S. handles of approved quality. Inner face of cash counter to be provided with 16G, M.S. sheet, painted with two coats of synthetic enamel paint of approved shade & make, over a coat of approved steel primer. Inner faces shall be painted with two coats of synthetic enamel paint of approved shade & quality over a coat of primer. Skirting: 100mm high laminate of approved make & shade all as directed. | Rft | 4.50 |  |  |
| 4.2 | GLASS BARRIER ON CASH CABIN / OFFICER COUNTER Providing & fixing 12mm thick clear float glass on Cash Counters at front & sides, with S S clamps/patch fittings, necessary polished beadings etc complete all as directed by the Bank. Note: Front glass to be fixed in slanting position towards inside cash area with required angle as directed. Above glass to be fixed either in two pieces. Lower piece having one semi-circular/rectangle 6” dia/long opening at customer top level & other piece to be fixed at suitable height making 3” slit for conversation with customer. In this case, only glass area to be measured for payment purpose.) In case of single piece, one semi- circular/rectangle 6” dia/long opening at customer top level & one circular hole of required dia at suitable height for conversation at centre of cash counter to be provided. | Sqft | 45.00 |  |  |
| 4.4 | OFFICER COUNTERS / LOW LEVEL COUNTER Supplying and erecting counters (running tables) 915 mm wide and 750mm high including drawer-cum- cabinet unit for files, space for computer CPU, wire managers, modesty panel, tea tray etc. made with 19mm thick BWP grade block board on top and supporting sides / front with necessary well seasoned Malaysian Sal/ Kail/ Mirandi wood frame work. Writing top & customer top should be finished with mirror polished; 18mm thick granite slab (Jet black or shade as directed by Bank's Engineer) with rounded bevelled edges and sides with 1.0mm thick laminate of approved shade/make finished on sides with 1.0 mm thick laminate of approved shade and quality, including providing and fixing 450mm long key board tray of approved shade, hardware and accessories at each of the counter. All exposed edges of block board are to be provided with half round teak wood mouldings, duly polished and 100mm high laminate of approved make & shade all as directed. Customer Top will be raised by 6 inches to provide a box type formation in the front elevation. Inner faces shall be painted with two coats of synthetic enamel paint of approved shade & quality over a coat of primer. Skirting: 100mm high laminate of approved make & shade all as directed. Rate to include charges for brass hardware fittings as well as fixtures like sliding gear for drawers of ‘Earl Behari’ or equivalent approved make. (ELEVATION DESIGN OF COUNTER WILL BE PROVIDED BY ARCHITECT AS PER SITE CONDITION) | Rft | 22.50 |  |  |
| 5.0 | DOORS : |  |  |  |  |
| 5.1 | Supplying and fixing in position overall 32 mm thick solid core hot pressed phenol formaldehyde bonded flush door finished with 1.0 mm thick laminate of approved shade and quality on both sides of the door shutter & beading on edges including cost of hardware fittings e.g. Godrej night latch / mortise lock, hinges, door closure, handles etc. of approved make and quality. (CASH CABIN ) . Please note that the price must include providing and fixing aluminum grill with square size 75mm x 75 mm in the door as per Bank’s instructions with grill tightly stretched and nailed to wooden framework along with steam beach wood beads of approved design complete for Cash Cabin Door as per Bank’s instructions | Sqft | 21 |  |  |
| 5.2 | Supplying and fixing in position 32 mm thick solid core hot pressed phenol formaldehyde bonded partly glazed flush door shutter of approved make finished up to 900 mm from floor level with 1.00 mm thick laminate of approved quality on both sides of the door shutter & beading on edges. The upper portion of the shutter is to be provided with 12 mm thick clear float glass fixed with white or steam beech wood glazing beads of approved design, complete as per drawing. Rate to include cost of hardware fittings e.g. mortise lock, hinges, handles, door closer etc. of approved make and quality. (BM CABIN DOOR / PASSAGE DOOR / STR ROOM PASSAGE DOOR ) | Sqft | 42.00 |  |  |
| 5.3 | FLAP DOOR Providing and fixing Flap Door of min. size: 3’ (B) X 2’6” (H) made of block board with approved lamination on both side, of required size with hinges/tower bolts/stopper/handles etc complete. | Sqft | 36.00 |  |  |
| 5.4 | ENTRANCE DOOR - TOUGHENED GLASS DOOR WITH FIXED GLASS PARTITION & ETCHING WORK ( 2 UNITS) (DOUBLE DOOR IN EACH UNIT) Providing fabricating & fixing in position twin –style door at Main Entrance made of 12mm thick clear toughened glass (MODI / ST. GOBAIN) in single piece with computerized diamond cut and edge polish pencil edges all around including making required cut holes for lock, handles, pivot patch fitting, corner fitting , hinges , etc. Glass should be fixed in S.S finished D-Brackets, etc. Wherever required with metal patch fittings at Top / Bottom / Corner / profile section for fixed glass as approved by Bank Authorities for Door Shutter. For Glass Partition wherever required if any, the glass panels can be bolted together with stainless steel stub post which are into concrete through core drill holes or supported by stainless steel clamp fixing to vertical posts on each side as per requirement at the site with necessary bevelling etc. complete fixed on floor spring of Ozone / Dorma / Godrej Make ( ISI Mark & Heavy Duty). The Door Shutters shall have C - shaped Stainless Steel Handle ( 18" or 24" as directed ) on both the door and both the sides of approved make with erection, finishing, etc complete. The fixed glass partition are to be fixed by providing profile section @ Top & Bottom . The entire glass partition is to be finished with one way film, bank logo or as per Bank's Requirement. | Sqft | 70.00 |  |  |
| 6.0 | OTHER ITEMS |  |  |  |  |
| 6.1 | LOW / FULL HEIGHT STORAGE CABINET P/F in position 450mm deep storage full ht./low ht.cabinets having sides partitions,shelves & shutters,fabricated out of 19mm BWR & 6mm th.BWR Plywood back,with all exposed surfaces & inside of shutters/internal base of drawers to be finished with 1mm thick laminate i/c 31x31mm rounded steem beech wood moulding & 6x19mm steem beech wood edging on edges of comm.board as/arch.drg.complete with shelves,shutter,drawers etc.i/c P/F necessary hardware telescopic channels for drawers of Hettich,S.S.piano hinges as/appvd.sample,Hettich handles(as/appvd.sample)in all drawers & shutters,locking arrangement,75mm brass tower bolts,Hettich locks in all shutters & drawers,magnet catcher,painting the exposed internal surfaces of block board with 2-3 coats of synthetic enamel paint with melamine finish on wood surface with 6mm comm.ply at rear as per arch.drg.& as per site requirements the rate should include 19mm th.pre-polished Granite Stone (Asian Quartz) as per appoved sample with half round bevelled edge complete on low ht.storage cabinets. | Sq.Ft. | 100.00 |  |  |
| 6.2 | MAIN PANEL BOXING |  |  |  |  |
|  | Providing and fixing 1'4" deep electrical panel cabinet. The basic structure of cabinet includes dividers and shelves made up of 18 mm thick Bison panel sheet. It shall have openable shutters with louvers and shall be made with 2" x 1" x min 1.8mm thick aluminium section framework covered with 6mm bison panel sheet on all sides with 16 Gu.powder coated performated S.S. Sheet as infill material. It will have 1mm laminate on outside surface as per Architect & Bank's Instructions. The entire unit shall finish with paint on internal surface. The rate includes all the hardwares, labour, scaffolding, etc complete | Sq.Ft | 25.00 |  |  |
| 6.3 | WOODEN PANNELING ON FRONT / ROLLING SHUTTER COLUMN |  |  |  |  |
|  | Providing and fixing in required position 4mm thick exterior grade Aluminium Composite Panel in Ceiling and Vertical Cladding or as per requirement and directed by Bank's Engineer or Architect over a framework made out of 50 x 50 x 2 mm aluminium SHS anchored to existing M.S Structure including installing the composite panel in grid pattern of 2' x 2' or of required size as directed with the help of aluminium cleats / angles of size 1" x 1" and striking out joints with weather silicon. The rate shall include cutting of panels, its bending, cutting for providing down lighters, providing opening for operation of rolling shutters, labour, scaffolding, etc complete | Sq.Ft | 50.00 |  |  |
| 7.0 | FIXTURES |  |  |  |  |
|  | OFFICER TABLE WITH SIDE CREDENZA WITHOUT GLASS Providing and placing in position tables with top, vertical sides/dividers, skirting and modesty panel made out of 19mm thick water proof board with 1mm thick laminate (as/shade approved by the Architect) on all exposed surfaces. It shall also have 75mm x 40mm teak wood runners below the top and for footrest below. Every table shall be provided with a side unit (size – W-500mm, H-750mm, D-500mm approx.), consisting of 150 mm deep drawer at top and a cabinet with open able/hinged shutter below. The facia of drawer and shutter of cabinet below shall be made out 19mm thick water proof board with 1mm thick laminate (as/shade approved by the Architect) and sides and bottom of drawer shall be made out of 12mm thick water proof ply. The side unit shall have CAM locks, handles, guides, sliding units, hinges, magnetic catcher’s etc. of approved make. Every table shall be provided with a pedestal unit (size – W-550mm, H-720mm, D-450mm approx.) below. The table shall have 100mm deep facia, below the top, in which keyboard drawer unit shall be adjusted. It shall be made out 19mm thick water proof board with 1mm thick laminate (as/shade approved by the Architect) and sides and bottom of drawer shall be made out of 12mm thick water proof ply. The unit shall have CAM locks, handles, guides, sliding units etc. of approved make. The job shall include for 6mm x 6mm grooves in between sides and open able drawers and hinged shutters. All the exposed edges of water proof board and ply shall be provided with 6mm thick teak wood/hard wood lipping. All teak wood surfaces shall be melamine polished and other internal surfaces shall be painted with two coats of synthetic enamel over one coat of primer, all complete to the satisfaction of the Architect. Every table top shall be provided with 12 mm thick float glass (as per profile) with polished and beveled (25mm wide) edges all complete to the satisfaction of the Architect. Approx size of table : (4'.6"’x 2'6"’x 2.5’) | No. | 1.00 |  |  |
| 7.1 | BRANCH MANAGER TABLE WITH SIDE CREDENZA INCLUDE 12mm THICK GLASS:- Providing and placing in position tables with top, vertical sides/dividers, skirting and modesty panel made out of 19mm thick water proof board with 1mm thick laminate (as/shade approved by the Architect) on all exposed surfaces. It shall also have 75mm x 40mm teak wood runners below the top and for footrest below. Every table shall be provided with a side unit (size – W-500mm, H-750mm, D-500mm approx.), consisting of 150 mm deep drawer at top and a cabinet with open able/hinged shutter below. The facia of drawer and shutter of cabinet below shall be made out 19mm thick water proof board with 1mm thick laminate (as/shade approved by the Architect) and sides and bottom of drawer shall be made out of 12mm thick water proof ply. The side unit shall have CAM locks, handles, guides, sliding units, hinges, magnetic catcher’s etc. of approved make. Every table shall be provided with a pedestal unit (size – W-550mm, H-720mm, D-450mm approx.) below. The table shall have 100mm deep facia, below the top, in which keyboard drawer unit shall be adjusted. It shall be made out 19mm thick water proof board with 1mm thick laminate (as/shade approved by the Architect) and sides and bottom of drawer shall be made out of 12mm thick water proof ply. The unit shall have CAM locks, handles, guides, sliding units etc. of approved make. The job shall include for 6mm x 6mm grooves in between sides and open able drawers and hinged shutters. All the exposed edges of water proof board and ply shall be provided with 6mm thick teak wood/hard wood lipping. All teak wood surfaces shall be melamine polished and other internal surfaces shall be painted with two coats of synthetic enamel over one coat of primer, all complete to the satisfaction of the Architect. Every table top shall be provided with 12 mm thick float glass (as per profile) with polished and beveled (25mm wide) edges all complete to the satisfaction of the Architect. Approx size of table : (5'6" x 2'6"’x 2.5’) | No. | 1.00 |  |  |
| 8.0 | MISCELLANEOUS ITEMS |  |  |  |  |
| 8.1 | SLIP & CUSTOMER WRITING DESK | No. | 2.00 |  |  |
|  | Providing and fixing in position (with rear wall) slip and customer writing desk (as per design) of overall size – 1200mm x 450mm x 300mm. The fixing shall include for expansion bolts, cleats, clamps, bolts, nuts, screws, rivets and other anodized accessories, all complete up to the satisfaction of the Architect. |  |  |  |  |
|  | The writing top and facia/sides shall be made out of 19mm thick water proof board with 1mm thick laminate (as per shade approved by the architect), on all sides and . It shall be laid over the base of 19mm thick water proof board (already considered in various items.) and fixed with araldite. It shall include for beveling and polishing at all the exposed edges. The rate shall include cost of all materials, labour, scaffolding, wastages, polishing and T& P etc. It shall be made out of 12mm thick water proof plywood (two surfaces of 6mm thickness, fixed together, where required) on rear and both sides. The front shall be made out of 12mm thick glass (as per profile). It shall be fixed in the grooves, made in side members, in an inclined manner (as per profile in drawing enclosed) with a gap of 75mm (max.) in between. The space shall be further divided in four equal parts with help of 12mm thick water proof plywood. All edges of the glass shall be polished. All the exposed edges of water proof board and plywood shall be provided with 6mm thick teak wood/hard wood lipping. All teak wood surfaces shall be melamine polished and other internal surfaces shall be painted with two coats of synthetic enamel over one coat of primer, all complete to the satisfaction of the Architect. |  |  |  |  |
| 8.2 | CHEQUE DROP BOX / SUGGESTION BOX | No. | 2.00 |  |  |
|  | Supplying and erecting cheque drop box / suggestion box (over all size 400mmx150mmx600mm) made of 19 mm thick BWP grade ply board and curved portion at top made out of 90mmx90mm C.P. teak/steam beach wood section. A slit opening of 10mmx150mm is to be provided at the top of the box. An openable shutter of 259mmx262mm size made of 19 mm thick BWP grade block board is to be provided. |  |  |  |  |
|  | The entire exposed timber surface including all free edges is to be finished with 1mm thick laminate sheet of combination shade. Inner faces shall be painted with two coats of synthetic enamel paint of approved shade & quality over a coat of primer. Rate to include cost of necessary hardware fittings e.g. hinges, lock, knob, etc. of approved design and quality. |  |  |  |  |
| 8.3 | NOTICE BOARD | No. | 1.00 |  |  |
|  | Providing and fixing in position notice board (as per design) of size – 900mm x 600mm x 150mm. The sides shall be made out of 19mm thick water proof board with 1mm thick laminate (as per shade approved by the architect) on all exposed surfaces.It shall have 12mm thick tack The front shall have 6mm thick float glass sliding shutter in two parts.It shall also have 19mm wide two track aluminum channel for sliding arrangement, all around, along with locking arrangement. Board (Make – Cellotex or equivalent) wrapped with stretched blazer cloth (Make- Raymonds), of Rs. 200.00 per Mt. (approx.) in colour and pattern approved by the Architect. It shall be fixed with help of kail/partal wood (sect. 19mm x 25mm) frame work. It shall be covered with 6mm thick water proof plywood, on the back. All the exposed edges of water proof board and ply shall be provided with 6mm thick teak wood/hard wood lipping. All teak wood surfaces shall be melamine polished and other internal surfaces shall be painted with two coats of synthetic enamel over one coat of primer, all complete to the satisfaction of the Architect. |  |  |  |  |
| 8.4 | ETCHING WORK & BANK LOGO FOR BRANCH | LS | 1.00 |  |  |
|  | Etching for Toughened glass, Etching for Front counters / Manager Cabin. |  |  |  |  |
|  | Etching work on glass with chemicals (as per design and profile) as and where required. The rates shall include cost of all materials, labour, T& P, wastage, transportation etc. required for proper completion of work. It shall be made out of following profile.-Bank’s logo (300mm dia. / 300mm x 300mm).All complete to the satisfaction of the Architect. |  |  |  |  |
| 8.5 | VERTICAL BLINDS | Sq.Ft | 75.00 |  |  |
|  | Providing & fixing in position vertical blinds of Vista as per approved colour by Bank’s Engineer & Architect. |  |  |  |  |
| 8.6 | PAINTING & FINISHING WORK |  |  |  |  |
| 8.6.1 | Painting on Walls and Ceilings with two or more coats of first quality of Acrylic Emulsion (Make & Shade Approved by Bank / Architect). It shall include for preparing the surfaces clean, even and smooth with thoroughly brushing with broom, scratching with patti and sand papering respectively, so that the surfaces are free from oil, grease, dirt, mortar droppings & other foreign and loose matter. It shall also include for preparing the surface and filling of holes with 2 coat of first quality acrylic wall putty (Birla Wall Care / JK / Asian) before priming coat of whiting / approved primer. The rate shall include cost of all materials such as plaster of paris, whiting /primer, glue (Fevicol), indigo / stainer, brushes, sand paper, patti, dhoti, broom stick, clean water with storage and other containers, labour, scaffolding / ladder charges, cleaning of floors and other sundries etc., all complete to the satisfaction of the Architect. Please note that in the Branch Hall, walls are to be painted below false ceiling and ceiling is not be painted. | Sq.Ft | 2000.00 |  |  |
| 8.6.2 | Providing & applying 2 coats of first quality synthetic enamel paint over a coat of Zinc Chromate metal primer to steel surfaces (window grill, collapsible gate) all of approve make and shade including surface preparation by complete scrapping the existing paint / rust on grills / shutter by using chemical paint striper or suitable approved method, cleaning, filling holes, smoothening the surface all complete as directed by Bank’s Engineer. (Measurement Coefficient for each side = 0.5) | Sq.Ft | 90.00 |  |  |
| 8.6.3 | Rolling Shutter as same as above in 8.6.2 but measurement shall be taken for each side with coefficient 1.10 each side. | Sq.Ft | 90.00 |  |  |
| 9.00 | Work of dismanteling of existing partitions, counter, electrical fiitings and other strcuture in the branch premises and arrangement of temporary setup of table, LAN cable, Lights etc. for functioning of branch during furnishing process. The temporary setup to be dismanteled and removed after coompletion of the renovation works. Bank will not pay additional amount for temporary setup done by the contractor for smooth funtcioning of the branch. | LS | 1.00 |  |  |
|  | RESIDUAL EXPENSES |  |  |  |  |
|  | FURNISHING WORK |  |  |  |  |
|  | GROSS AMOUNT |  |  |  |  |

| PROJECT - ESTIMATE FOR ELECTRICAL WORKS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BANK - INDIAN BANK |  |  |  |  |  |
| BRANCH - MILKIPUR |  |  |  |  |  |
| S.N | PARTICULARS | Unit | Qty | Rate | Amount |
|  | The work shall also include for cutting chase for conduits and moulded boxes, if required, in the wall/ floor (Without damaging the building/wall) along with clamping the conduit at every 750mm interval & finally finishing the surface & matching the same with the existing surface. Item 1,3, 4 & 5 |  |  |  |  |
|  | The rate should include the cost of all materials, modular switch complete with MS box & cover plate, labour, T&P, taxes & levies, scaffolding, shifting of furniture, carting away the debris and cleaning the floor etc |  |  |  |  |
|  | CABLING |  |  |  |  |
| 1 | DG SET SUPPLY & MAIN PANEL SUPPLY |  |  |  |  |
|  | Laying of existing 3.5c 25sq.mm XLPE insulated armoured Aluminium Conductor cable c/w 1*6sq.mm stranded copper wire for earthing from proposed DG set to Changeover switch including its connections at both ends with lugs to be laid in 25mm dia rigid PVC /flexible conduit | LS | 1 |  |  |
| 3 | S/ laying of XLPE insulated armoured Copper Conductor cables including cost of lugs, cable glands etc. The cable should be laid on surface with proper clamps @750mm or laid & concealed in wall/ floor/ PVC Conduit/ false ceiling complete in all respect. The work also includes cutting chases for conduits in the wall/ floor (Without damaging the building) along with clamping the cable. All connection shall be done thru proper size of lugs |  |  |  |  |
|  | 4c x 10sq.mm for TPN Db's | Metre | 20 |  |  |
| 4 | Incoming supply to UPS room: S/laying of 4C X 6sq.mm XLPE insulated armoured Copper conductor cable from main Panel to 4way SPN Db/ 4way TPN Db in the UPS room to be laid in medium duty 20mm PVC conduit/ pipe & run on surface or concealed in wall/ floor/ flase ceiling including connections at both ends with lugs. | mtr | 30 |  |  |
| 5 | Supply & Laying of 4 runs of 6.0 Sqmm and 2 runs of 2.5 sqmm, PVC insulated, single core, multistranded copper conductor cable in suitable rigid / flexible metal conduit from MainUPS DB to auto phase shifter. | mtr | 20 |  |  |
| 6 | Supply & Laying of 2 runs of 6.0 Sqmm and 1 run of 2.5 sqmm, PVC insulated, single core, multistranded copper conductor cable in suitable rigid PVC conduit from automatch phase shifter to UPS unit | mtr | 20 |  |  |
| 7 | Wiring with 3*4.0sq.mm FRLSH stranded Cu. wires laid in 20mm OD PVC/ flexible pipe heavy duty laid on floor & concealed in wall/ floor as per requirement to various voltage stabilizers. The job includes making connections at both ends with proper size of lugs to the entire satisfaction of the Bank. | mtr | 25 |  |  |
| B | DISTRIBUTION MAINS |  |  |  |  |
| 1 | Supply, fabricating,assembling,internal- connection, testing & connceting with incomming, out going power cables & lines as required to complete the job of cubical type electrical panal made from 14SWG CRC. M.S. sheet with proper earthing terminal(Nut- bolt &washer) 02 No.outside of panal, same nut- bolt to tighten with internal earth busbar of panal situated inside loose wire box(on top portion of panel) On top side of panel cable entry arragement to be done for incomming & out going of lines. Painting powder coating electrical grey/ seimensgrey to be done.one 6"x4" enamal type 440Volt Danger board to fix up on main panal bus-bar cover, components are as follows :-All switches are front handle operated & all MCB knob should be visual, all component to provide in separate compartment only DBs in one compartment. | Each | 1 |  |  |
|  | Incomer - 100A 4P MCCB 25Ka with adjustable thermal & Magnetic trip unit -1 Nos |  |  |  |  |
|  | OUTGOINGS - |  |  |  |  |
|  | 63 Amps 4 pole MCB. = 04 Nos.(AC & Power DB,ATM ,UPS,spare) Amps Meter 0-100 Amps with 3 Nos. CT & selector switch 96x96mm 01 Set (Changeover should be ON-LOAD type.) |  |  |  |  |
|  | 100Amps 4pole Cu. Bus Bar. 01 Set. 100Amps 4 pole Change over switch. 01 No. 100 Amps FP MCCB 25KA as Gen - Set main incoming 01 NO. |  |  |  |  |
|  | Volt meter 0-500volt A.C. 50 Hz.96x96mm with selector switch. AE make.n 01 Set |  |  |  |  |
|  | 20mmdia LED indicator lamp 03 Nos. |  |  |  |  |
|  | 02 Amps Sliding Fuse. 06 Set. |  |  |  |  |
|  | 150x100mm enamal Danger board 440 volt. |  |  |  |  |
|  | (to set on bus bar cover). 01 No. |  |  |  |  |
|  | Cable entry from top & bottom - removable gland plates (bottom at 2 points) c/w powder coated Grey paint (two coats) with name labels for all incoming & outgoing feeders. The panel shall be complete in all respects including all accessories such as earthing bus, Air ventilators, lifting hooks etc. |  |  |  |  |
| 2 | AC DB. Make standard/Legrand/ABB ,Double door | Set | 1 |  |  |
|  | 4WAY TPN DB with 4P MCB as incoming |  |  |  |  |
|  | 63 TP&N MCB incomer |  |  |  |  |
|  | 25Amps SP MCB - 4 Nos ( branch Ac :4 Nos ) + 16AMPS SP X1 NOS. ( BM AC ) 25Amps SP MCB - 3 Nos (RAW power ) + 32AMPS DP - 1no. -LED Sign board (branch) + 32 AMPS DB MCB 1No: For Strong room |  |  |  |  |
|  | including Neutral Bar. |  |  |  |  |
| 3 | LIGHTING DB Make standard/Legrand/ABB | Set | 1 |  |  |
|  | 4 way TPN MCB DB with TP MCB as incoming |  |  |  |  |
|  | 40A TP&N MCB as main |  |  |  |  |
|  | 16 Amps SP MCB - 03 Nos.+ (BM +Toilet) each 1No + 1No spare |  |  |  |  |
|  | 12 Amps SP MCB - 9 Nos. (Hall - 4Nos + Store room 1 No +Exteranl Lighting 1No.:+ wall Fan 2 Nos + 1 ATM Lights) |  |  |  |  |
|  | Including Neutral Bar. 01 Set |  |  |  |  |
| 5 | UPS SYSTEM (Branch) |  |  |  |  |
| 5.1 | UPS Main DB Supply, erection, testing and commissioning of three phase 12Way SPN MCB DB double door type with "C" Curve MCBs, IP 42 powder coated for UPS with 63A 4P MCBs – 1No as INCOMER 40A TP MCBs – 2 Nos. as OUTGOING (UPS :1Nos , 1 set - Spare) Including all interconnections & accessories etc Make standard/Legrand/ABB | Nos | 1 |  |  |
| 5.2 | Supply & Fixing of 32A DP MCB in 2way SPN Db IP 20 metallic directly on wall Raw power Incoming & OutGoing supply for UPS (Branch :2 Nos & ATM 1No.) | Each | 2 |  |  |
| 5.3 | UPS Output Db 1 |  |  |  |  |
|  | S/F of 6way SPN Db c/w 32A DP MCB as incomer 2*16A 2P MCB's for FDAS & CCTV + 1*10A 1P MCB for digital signage + 1*10A spare, including proper connections thru lugs. The job shall be complete & to the entire satisfaction of the Bank. All DB's to be properly earthed. | Each | 1 |  |  |
| 5.4 | UPS Output Db - 2 |  |  |  |  |
|  | S/F of 12way SPN Db for branch computer points with 32A 2P MCB as incomer and 3*10A SP MCB for 1 BM & 2 for 4 officers (2 on LHS counters o/s BM & 2 besides cash) + 2*6A SP for 2 cash cabins + 2*10A 1P MCB for router + 3*10A spare complete in all respect and as required & to the satisfaction of the bank. All DB's to be properly earthed. | Set | 1 |  |  |
|  | SPN Db should be earthed properly thru lugs. |  |  |  |  |
| 5.6 | Supply, installation, testing and commissioning of 40A rated 3 phase to single phase auto phase shifter for 3.0 KVA UPS (SLHD of Swift make) (BRANCH M) | Nos | 2 |  |  |
| 1 | POINTS & WIRING |  |  |  |  |
| 1.1 | Supply & Installation of concealed Point Wiring for light, fan & exhaust fan points controlled thru modular switches complete with boxes & cover plate - The point wiring shall be carried with 3 x 1.5sq.mm FRLSH (Fire Resistant low smoke Halogen) stranded Cu. Wires in 20mm PVC conduit pipe concealed in wall/ floor/ panelling/ partitions or above false ceiling as per requirement. The job shall be complete & to the entire satisfaction of the Bank. Each Switch board should be provided with suitable capacity fuses/ fuse indicatort. All switch & sokets shall be modular type complete with cover plate & GI/ MS box. The work shall also include for cutting chase for conduits and moulded boxes, if required, in the wall/ floor (Without damaging the building/wall) along with clamping the conduit at every 600mm interval & finally finishing the surface & matching the same with the existing surface.The rate should include the cost of all materials, labour, T&P, taxes & levies, scaffolding, shifting of furniture, carting away the debris and cleaning the floor etc.The job shall be complete & to the entire satisfaction of the Bank. |  |  |  |  |
| a | Light points + AFF points including the cost of 6A switch | Point | 35 |  |  |
| b | Ceiling fan point + exhaust fanwith step type electronic regulator controlled by one 6 amp. Modular switch. The switch should be at switchboard level. | Point | 2 |  |  |
| c | Similar as above for wall mounting fan including box with 6A Control Switch along with clamp and ceiling rose | Point | 5 |  |  |
| d | 6A switch & Socket on SDb | Point | 11 |  |  |
|  | UPS POINTS |  |  |  |  |
|  | Supply & Installation of point wiring for UPS power plug points on workstations / table for computers using 3*1.5 Sq.mm copper conductor multi stranded FRLSH wire pulled through floor raceways/conduit and taken upto table top using PVC rigid or flexible conduits run within wooden/metal partitions. Each point consisting of 3 nos of 6A , 2/3 pin sockets & 1 No., 16A Switch wired together forming one point. The earth wire of green color only. | Each | 8 |  |  |
| 1.2 | RAW POWER POINT |  |  |  |  |
|  | Single Power Points - Supply & Installation of Circuit Wiring (Single 6/16Amps power Points) : Supply & laying of circuit wiring for 6/16 Amp. Switch points with 2 x2.5 + 1x 1.5sq.mm FRLSH (Fire Resistant low smoke Halogen) Cu stranded wires laid in 20mm conduit pipe concealed in wall/ floor/ panelling/ partitions as per requirement. The job shall be complete & to the entire satisfaction of the Bank.t. The earth wire of green color only. All switch & sokets shall be modular type complete with cover plate & GI/ MS box. The work shall also include for cutting chase for conduits and moulded boxes, if required, in the wall/ floor (Without damaging the building/wall) along with clamping the conduit at every 600mm interval & finally finishing the surface & matching the same with the existing surface.The rate should include the cost of all materials, labour, T&P, taxes & levies, scaffolding, shifting of furniture, carting away the debris and cleaning the floor etc.The job shall be complete & to the entire satisfaction of the Bank. | Point | 5 |  |  |
| 1.3 | AC POINTS |  |  |  |  |
| 1.3 | AIR CONDITION POINTS -Circuit wiring with 2x4.0sq.mm + 1x 1.5sq.mm FRLS Cu stranded wires (Finolex/ Anchor/ Skytone) to be laid in 19mm PVC pipe heavy duty laid & concealed in wall/ floor/ panelling/ partitions as per requirement. The job includes Supply & Fixing of Havells MCB protected power unit DBOXx type c/w 32A DP MCB including proper connections or any equivalent approved by Architect. The job shall be complete & to the entire satisfaction of the Bank (FOR CASSATE / SPLIT AC 2.0 TR UNITS) | Point | 5 |  |  |
| 1.4 | STRONG ROOM MCB |  |  |  |  |
|  | Strong room controlling DP MCB at entry gate outside of strong room - Circuit wiring with 2x2.5sq.mm + 1x 1.0sq.mm stranded Cu. conductor (Finolex/Anchor/Havells) to be laid in 19mm M S conduit pipe heavy duty 16swg laid & concealed in wall/ floor/ paneling/ partitions or above false ceiling as per requirement. The electric wiring should be arranged separately for the Strong Room and it is to be connected to a plug outside. The job also includes supply & fixing of 32A DP MCB in 2way box including connections with lugs. The rate should include the cost of all materials, labour, T&P, taxes & levies, scaffolding, shifting of furniture, carting away the debris and cleaning the floor etc.The job shall be complete & to the entire satisfaction of the Bank. | Job | 1 |  |  |
| 6 | Glow sign Board - Circuit wiring with 2x2.5sq.mm + 1x 1.5sq.mm stranded Cu. conductor (Finolex/Anchor/Skytone) to be laid in 20 mm PVC pipe heavy duty laid & concealed in wall/ floor/ panelling/ partitions or above false ceiling as per requirement. The rates should include the cost of all materials, labour, T&P, taxes & levies, scaffolding, shifting of furniture, carting away the debris and cleaning the floor etc.The job shall be complete & to the entire satisfaction of the Bank. | JOB | 1 |  |  |
| 7 | EARTHING JOB |  |  |  |  |
| 7.1 | Supply, Installation, Testing and Commissioning of Maintenance Free Earthing system made up of Copper bonded rod / Galvanized JK Chemrode 10 feet length, 25 mm dia. (Minimum copper bonding shall be 0.25mm) filled with highly conducting metallic compounds with permanent sealing at both the ends with lead terminal including bore well, chamber with cover and all required civil allied works, earth pit to be filled with 2 bags of 25 kg pack of Back Fill Compound (Earth Enhancement Material). The rate should be inclusive of Rod-to Conductor connectors, Earth enhancement material, Pit Cover and other accessories as required and as per specification and other applicable codes (include chamber for earthing, Earthing certificate to be submitted along with the bill). The job also includes wiring with 1 x 10 sq.mm PVC insulated copper conductor wire of 650 / 1100 Volt grade (Finolex, Polycab Make) upto 10 m as earth wire with concealed / exposed GI pipe including making all the connections with guaranteed E/N voltage of less than 3 volts and resistance less than 1 ohms. | Nos | 2 |  |  |
| 7.2 | Supplying, laying and fixing of 10 sq mm copper pvc insulated wire covered with GI Pipes from earthing plate to main panel, if required beyond 10m | Metre | 20 |  |  |
| 8 | F - FIXTURES & FITTINGS |  |  |  |  |
| 8.1 | Supplying and fixing PHILIPS / CROMPTON / HAVELLS make 36W LED FLAT PANEL after disconnection of existing fitting 2'x2' size in grid type false ceiling, 600x600 suitable for recess mounting,CRCA sheet, asthetic looking, complete. | Each | 14 |  |  |
| 8.2 | Supplying and fixing Philips make or eqv 15 W LED round type downlighter (Philips / Crompton / Havells / Wipro / Jaguar) | Each | 5 |  |  |
| 8.3 | Supply installation testing and commissioning of 1*20watt Batten type tubular LED light fitting with LEDs, driver, diffuser, and housing complete in all respects in UPS room and record /wc/bath room /suitable place with complete in all respect with required color on need basis. | Each | 11 |  |  |
| 8.4 | S/F of angle holder for bulbs including cost of 12W CFL lamp for Toilets | Each | 2 |  |  |
| 8.5 | Supply & installation of wall bracket fan 450 mm Metal body (Heavy duty/ high speed) Make Cropmton Greaves / Havells/Orient (to be supplied after consultation with architect/Electrical Engineer as per approved drawing) | Each | 6 |  |  |
| 8.6 | Supply & fixing of the 1200 mm 48") Sweep Ceiling fan (Orient / Crompton Greaves/ Havells) with downward rod(below false ceiling), canopy etc. including of PVC flexible copper wire.(white colour)( High speed) (Warranty Cerificate is necessary)with downward rod, circular boxes, ball sockets, ceiling plates,suitable arm bracket etc. including connection with wire)White colour (WITH REGULATOR) | Each | 2 |  |  |
| 8.7 | Supply & Fixing of fresh air fan 12" - Crompton Greaves / Usha/ POLAR / Bajaj | Each | 2 |  |  |
| 8.8 | S/F Heavy Duty Exhaust Fan 450mm with all the accessories complete as per Bank’s Instructions in UPS room | Each |  |  |  |
| 8.9 | Supplying and fixing High Luminius LED strip 3000 K 24 watt 5 mtr length with driver (Philips or Havells or Cormpton ) | SET | 10 |  |  |
| 10 | I - DATA CABLING |  |  |  |  |
| 10.1 | Supply & Fixing of XL RJ 45 toolless information outlet with Duplex face plate with dual connectors Grey colour cat 6e including cost of supplying & laying CAT 6e (as directed by Architect from Server to Switch, Switch to computer I/O data socket as required complete in all respects) in 25mm dia rigid PVC conduit (including laying of GI draw wire complete with junction box, circular box, elbows, bends & other accessories as required by chase cutting on floor, wall, through partition or over false ceiling.The data cabling pipe should be laid at a minimum distance of 3ft from pipes laid down for power distribution) including interconnection with connectors as required complete in all respects. | Each | 8 |  |  |
| 10.2 | Supply & fixing of 7ft Grey colour Patch chord Giga speed Systimax/ D link make | Each | 8 |  |  |
| 10.3 | Supply & fixing of 3ft Grey colour Patch chord Giga speed Systimax/ D link make | Each | 8 |  |  |
|  | ELECTRICAL WIRING WORKS |  |  |  |  |
|  | ELECTRICAL FIXTURES |  |  |  |  |
|  | LAN |  |  |  |  |
|  | GRAND TOTAL AMOUNT |  |  |  |  |
|  | *ALL RATES ARE EXCLUSIVE OF GST |  |  |  |  |

| S.NO. | SUMMARY | AMOUNT (RS.) |
| --- | --- | --- |
| 1. | CIVIL & INTERIORS WORKS |  |
| 2. | ELECTRICAL WORK (INCLUDING LAN CABLING) |  |
| 3. | GRAND TOTAL (RS.) |  |

Amount in Words: Rupees ……………………………………………………............ only.

GST Will be charged extra.

Place:

Date:					Signature & Seal of Tenderer

LIST OF APPROVED MAK - FURNISHING WORK

| Sl.No. | Materials | Approved manufacturers/suppliers |
| --- | --- | --- |
| 1 | Paint | I.C.I. ( Dulex - Duco) / Asian paints / Goodlac Nerolac |
| 2 | Plywood | BWR- Century/Green/Archid/Corbelttonly Premium quality |
| 3 | Wood preservative | BWR- Century/Green /Archid/Corbelttonly Premium quality |
| 4 | False ceiling | Armstrong, Indian gypsum board |
| 5 | Laminates | Sunmica / Archid / Century / Green/Corbeltt 1mm thick as per bank approved color |
| 6 | Screws | G.K.W./Nettle fold |
| 7 | Adhesive | FevicolSH |
| 8 | Night latch | Godrej |
| 9 | Locks | Godrej |
| 10 | Aluminum door frame | Jindal |
| 11 | Glass | Modi/Asahi/Saint-Gobain |
| 12 | Aluminum cladding | Eurobond/Alto bond/Alukbond |
| 13 | Keyboard drawer | EBCO |
| 14 | Telescopic CPU stand | EBCO |
| 15 | Cable organizer (wire manager) | EBCO |
| 16 | Vitrified tiles | Jhonsions Perlto |
| 17 | Venetion blind | Vista, Luxaflex |

LIST OF APPROVED MAKE – ELECTRICAL

| Sl. No. | Materials | Approved manufacturers/suppliers |
| --- | --- | --- |
| 1 | Cable Electrical | Finolex, Polycab, RR, Havel’s or Equivalent all wires used should be FRLS ( Flame Retardant , Low Smoke Low Halogen (FRLS) |
| 2 | Cable Telephone | D-Link, AT&T, Anchor, Deltron, or Equivalent |
| 3 | DataCableCAT6 | D-Link, Legrant, Systemac, Polycab, Finilex, Digisole or Equivalent |
| 4 | Glands & Lugs | Dowells, L&T, Cosmos or Equivalent |
| 5 | Conduits(PVC) | Supreme, Diamond, Astral, Polypast (PVC), Plasto, Precision or Equivalent |
| 6 | Conduits(GI) | Tata, Jindal or Equivalent |
| 7 | Casing Caping & Accessories | Ajanta, Modi, Asian, Precision or Equivalent |
| 8 | Switches | Legrand, Anchor, Havells, Roma, GM, L&T, Schneider electric, C&S electric, Orpat, MK or Equivalent |
| 9 | DB Panels ,MCB ,MCCB, ELCB, TPN & their Fittings | Siemens, Havells, Legrande, L & T, Schneider electric, Hagor, ABB or Equivalent |
| 10 | Exhaust Fan | GM, Crompton Greaves, KEI, Usha, Havells, Bajaj, Orient, Almonard, Alstom or Equivalent |
| 11 | Ceiling Fan, Wall Fan | GM, Crompton Greaves, Usha, Havells, Bajaj, Orient, Almonard, Alstom or Equivalent |
| 12 | Ceiling rose | Anchor, Precision, Rider. GM, or Equivalent |
| 13 | Capacitors | Crompton, Greaves, Asian, L&T, Havells or Equivalent |
| 14 | LED Luminaires all types | Philips, Syska Regent, Wipro, osram, Bajaj, Havells, Haionix, Panasonic, Goldmedalor Equivalent |
| 15 | UPS Battery | Amron, Rocket, Exide, Luminous, Panasonic, Okaya or Equivalent |
| 16 | Speakers | Philips, Ahuja Bose, JBL, Harman, Kardon, Sony, BlauPunkt, Panasonic or Equivalent |